Assignment: Human Sensory Thresholds

In this assignment, you will design and implement an interactive application to evaluate selected human sensory thresholds related to vision and hearing. The purpose of this assignment is to help you understand how human perceptual capabilities and limitations can be measured, interpreted, and applied in engineering psychology and human factors design.

This is an individual assignment. Each student should complete and submit their own work.

You may use AI-assisted development tools, such as Codex, Claude, or similar platforms, to support application design, coding, debugging, and documentation. If you use such tools, you must save and submit the complete interaction history, including prompts, responses, code suggestions, and any modifications you made based on the generated output, as part of your assignment materials. This requirement is intended to support transparency, reproducibility, and appropriate evaluation of your design and implementation process.

Engineering psychology examines how people perceive, process, and respond to information in designed systems. By developing and using sensory tests, you will explore how perceptual thresholds influence interface design, display readability, auditory feedback, warning signals, and human-system interaction.

For the scope of this assignment, you will focus on visual and auditory perception. The visual tests will examine contrast sensitivity and minimum distinguishable visual size. The auditory tests will examine pitch perception, temporal gap detection, pitch discrimination, and amplitude discrimination.

- Instructions

## Vision Tests

### 1. Contrast Sensitivity

Design a test inspired by the Pelli-Robson Contrast Sensitivity Test. Your application should display letters or symbols with gradually decreasing contrast and ask the participant to identify them. The test should continue until the participant can no longer reliably distinguish the stimulus.

Use the results to estimate the participant's contrast sensitivity as a percentage. Based on this value, discuss the approximate bit resolution required for a display system to represent perceptually meaningful differences. For example, an 8-bit display provides 256 discrete levels, corresponding to approximately 4% difference per level.

In your discussion, connect the result to engineering psychology topics such as display legibility, visual ergonomics, accessibility, and interface design under low-contrast conditions.

### 2. Smallest Noticeable Size

Design a visual acuity task inspired by the Tumbling E chart. Your application should display an E-like shape with equal stroke thickness and spacing between forks, where t = d = constant.

While the participant maintains a fixed viewing distance from the screen, gradually reduce the size of the shape until the participant can no longer distinguish the number or orientation of the forks. Use the screen dimensions, pixel resolution, and viewing distance to calculate the smallest distinguishable feature size.

Report the participant's approximate angular resolution in arc minutes, where 1 arc minute equals 1/60 of a degree. Discuss how this measurement relates to display design, symbol size, text readability, visual warnings, and viewing-distance requirements.

## Hearing Tests

### 3. Pitch Frequency Range

Design an auditory test that allows the participant to play tones between 20 Hz and 20 kHz using a slider or input field. Record the highest frequency that remains audible to the participant. The result should be accurate to at least the nearest 50 Hz.

You may also explore the lowest audible frequency, but this is not required because many consumer speakers do not accurately reproduce frequencies below 100 Hz.

Discuss the implications of the result for auditory display design, alarm systems, device feedback, and age- or equipment-related limitations in sound perception.

## Adaptive 3AFC Auditory Tests

The following three auditory tests should use a 3-alternative forced choice format, also known as 3AFC. In each trial, the participant should hear three options: two reference stimuli and one target stimulus. The participant must identify which option is different.

Use an adaptive threshold procedure, such as a 2-down 1-up method. In this method, the task becomes more difficult after two consecutive correct responses and easier after one incorrect response. The step size should gradually decrease during the experiment.

A reversal point occurs when the direction of difficulty changes, such as from harder to easier or from easier to harder. The test should end after the sixth reversal point. Calculate the final threshold by averaging the last four reversal points.

Suggested reading before implementing the 3AFC procedure: Use the following resources to understand adaptive psychophysical procedures, reversal points, and threshold estimation before coding the gap, pitch-difference, and amplitude-difference tests.

Adaptive methods overview: https://engineering.purdue.edu/~ece511/LectureNotes/pp12.pdf

2-down 1-up and psychophysical threshold background: https://www.pnas.org/doi/pdf/10.1073/pnas.081082598

Psychoacoustics and detection threshold reference: https://bdml.stanford.edu/twiki/pub/Haptics/DetectionThreshold/psychoacoustics.pdf

At the end of each adaptive test, your application should generate a plot showing:

Trial number on the x-axis

Stimulus difference or threshold parameter on the y-axis

Participant responses for each trial

The estimated final threshold

### 4. Sound Gap Detection

Create a test in which the reference options are continuous white-noise sounds of equal duration, while the target option contains a brief silent gap in the middle.

Use the adaptive 3AFC procedure to estimate the shortest gap duration the participant can detect. Report the threshold in milliseconds.

Discuss how temporal resolution in hearing relates to speech perception, alerts, auditory interface design, and time-critical feedback systems.

### 5. Pitch Difference Threshold

Create a test in which the reference options use a fixed pitch, such as 440 Hz or 1000 Hz, lasting approximately two seconds. The target option should contain a small pitch difference while maintaining the same duration and amplitude.

Use the adaptive 3AFC procedure to estimate the smallest detectable pitch difference. Report the threshold in Hz.

Discuss how pitch discrimination is relevant to auditory icons, alarms, music perception, notification design, and human-machine communication.

### 6. Amplitude Difference Threshold

Create a test in which the reference options use a fixed sound amplitude, while the target option contains a small amplitude difference. All sounds should have the same pitch and duration.

Use the adaptive 3AFC procedure to estimate the smallest detectable amplitude difference. Report the threshold in dB.

Conduct this test in a quiet environment and avoid using sound levels that are uncomfortable or potentially harmful.

Discuss how amplitude discrimination relates to volume control, auditory warnings, accessibility, and workplace sound design.

## Participant and Data Collection

Data may be collected from yourself or from a consenting volunteer. If the participant has pre-existing visual or auditory conditions or uses corrective devices, you may decide whether to test with or without those aids. Clearly describe the testing condition in your report.

All tests should be conducted in a controlled and respectful manner. Participants should be informed that the activity is for educational purposes only and that they may stop at any time.

## Important Disclaimer

These tests are intended strictly for educational demonstration in the context of engineering psychology. They are not medically validated diagnostic tools and should not be interpreted as professional medical evaluation, advice, or diagnosis. Anyone with concerns about their vision or hearing should consult a qualified healthcare professional.

Any information collected during this assignment should be used only for completing the course project. Do not permanently store, transmit, or share personal sensory data with external servers or third parties. The instructors and teaching assistants will only access the materials submitted through Canvas for grading purposes.

# Part I: Application Development

Develop an application that presents all required sensory tests, collects participant responses, records relevant test data, and calculates perceptual thresholds.

You may build either a mobile application or a web application. Your application should support clear instructions, consistent trial presentation, response recording, and result visualization.

Starter code: Before starting your implementation, review the provided Streamlit project structure here: https://github.com/priyanshum17/preceptual-sense-lab. You may use it as a starting point or adapt its structure for your own web or mobile implementation.

# Part II: Sensory Testing

Use your application to complete each sensory test. Record the necessary data for calculating thresholds and interpreting the results.

Save screenshots of the final result screen for each test, including the calculated threshold and any generated plots.

# Part III: Data Analysis and Report

In your lab report, describe how each test was implemented and conducted. Analyze the collected data and report the results for each sensory threshold.

Your report should also discuss whether your results are reasonable by comparing them with relevant literature or known ranges from perception research. Finally, explain how these sensory limitations may influence engineering design decisions, especially in visual displays, auditory alerts, interface accessibility, and human-system interaction.

# Resource Links

Adaptive psychophysics lecture notes: https://engineering.purdue.edu/~ece511/LectureNotes/pp12.pdf

Psychophysical threshold reference: https://www.pnas.org/doi/pdf/10.1073/pnas.081082598

Psychoacoustics reference: https://bdml.stanford.edu/twiki/pub/Haptics/DetectionThreshold/psychoacoustics.pdf

Starter application repository: https://github.com/priyanshum17/preceptual-sense-lab

# Submission Requirements

Submit the three separate materials on Canvas.

## File 1: Application Package

This file should include:

The completed application source code, including all files needed to run the visual and auditory sensory tests.

A brief README or instruction file explaining how to launch and use the application.

Any configuration files, dependency lists, or setup notes needed to reproduce your implementation.

Any raw or processed data files used to calculate the reported thresholds, if they are generated separately from the application output.

Submit this material as a compressed folder, such as a .zip file, unless otherwise instructed.

## File 2: Lab Report

This file should include:

Your name.

A brief description of how each sensory test was implemented.

A description of the testing procedure, participant condition, equipment, and environment.

Screenshots from the end of each test, including calculated thresholds and generated plots where applicable.

Results from all required visual and auditory tests.

Data analysis explaining how each threshold was calculated.

A discussion of whether the results are reasonable, supported by relevant literature or known perceptual ranges.

A clear explanation of how the results relate to engineering psychology, human factors, interface design, display design, auditory alerts, or accessibility.

Any limitations of your testing procedure, equipment, or data.

Submit this file as a Word document (.doc or .docx) or PDF.

## File 3: AI Interaction Transcript, If Applicable

If you used AI-assisted development tools such as Codex, Claude, ChatGPT, Gemini, Copilot, or similar systems, submit the relevant transcript as a separate file.

The transcript should clearly show:

Which AI tool or tools you used.

Your prompts or requests related to application design, coding, debugging, testing, data analysis, or writing.

The AI-generated responses, recommendations, explanations, or code suggestions.

Any follow-up questions or revisions you requested.

Any modifications you made to the generated output before including it in your final work.

# Suggested AI Tools

You may use any AI-assisted development tool for this assignment. Common options include:

Codex: https://chatgpt.com/codex

ChatGPT: https://chat.openai.com/

Claude: https://claude.ai/

Gemini: https://gemini.google.com/

Microsoft Copilot: https://copilot.microsoft.com/